Small-Scale Spiking Reservoir Network Design for Object Recognition Guided by Visual Cortical Processing Theory

Spiking Neural Networks (SNNs) are promising low-power alternatives to traditional Artificial Neural Networks (ANNs), which motivates research into their applications at the edge.

However, the design and training processes of SNNs are challenging given the temporal nature of neuromorphic computing, where typical training approaches such as backpropagation are difficult to implement.

One proposed solution to these issues is to build a Spiking Reservoir Network (SRN), a pool of sparsely interconnected neurons, where the strength and pattern of connectivity is randomly defined and training occurs only at the output layer.

This simplifies the design and training procedures but often necessitates the projection of input data to a high-dimensional space for effective separation of features at the output layer.

In this work, we investigate spatial and temporal dynamics in an SRN when stimulated with spatiotemporal patterns of visual data.

For high-resolution input data, the reservoir size can potentially be huge, which is motivation to develop SRNs with lower-dimensional spaces.

We leverage the twin-stream hypothesis of visual data processing in the brain to inform the design of a hierarchal, dual-pathway SRN, built from multiple sub-reservoir networks and consisting of fewer than 4,500 neurons in total.

Our method is evaluated on a novel object recognition dataset, and contrary to conventional SRN design, data is represented in a lower dimensional space relative to the spatial resolution at the input.

Furthermore, we demonstrate

We obtain consistently high accuracy on validation data using our design (μ = 95.2%, σ = 9.2 × 
10-3), demonstrating that the SRN performs well at the separation of spatiotemporal dynamics.